Cranes and Derricks in Construction

Shift and Monthly Inspection by a Competent Person 
29 CFR 1926.1412 and 29 CFR 1926.1413

CRANE: ___________________________________ DATE:

INSPECTOR: 	 SIGNATURE:

Check the box next to each item after it has passed inspection. Note any deficiencies or other observations that could pose a risk of injury or property damage. Equipment must not be used until all corrective action is taken. Keep this document for 3 months if using for monthly inspections.

|  | Item or Function Inspected | Deficiencies Found | Corrective Action Taken |
| --- | --- | --- | --- |
|  | Ground conditions around the equipment for proper support, including ground settling under and around outriggers/stabilizers and supporting foundations, groundwater accumulation, or similar conditions |  |  |
|  | Control mechanisms for maladjustments interfering with proper operation |  |  |
|  | Control and drive mechanisms for apparent excessive wear of components and contamination by lubricants, water, or other foreign matter |  |  |
|  | Air, hydraulic, and other pressurized lines for deterioration or leakage, particularly those that flex in normal operation |  |  |
|  | Hydraulic system for proper fluid level |  |  |
|  | Hooks and latches for deformation, cracks, excessive wear, or damage such as from chemicals or heat |  |  |
|  | Wire rope reeving for compliance with the manufacturer's specifications |  |  |
|  | Electrical apparatus for malfunctioning, signs of apparent excessive deterioration, dirt, or moisture accumulation |  |  |
|  | Tires (when in use) for proper inflation and condition |  |  |
|  | The equipment for level position within the tolerances specified by the equipment manufacturer's recommendations, both before each shift and after each move and setup |  |  |
|  | Operator cab windows for significant cracks, breaks, or other deficiencies that would hamper the operator's view |  |  |
|  | Rails, rail stops, rail clamps, and supporting surfaces when the equipment has rail traveling |  |  |
|  | Safety devices and operational aids for proper operation |  |  |
|  | Note: Where a wire rope is required to be removed from service, either the equipment (as a whole) or the hoist with that wire rope must be tagged-out until the wire rope is repaired or replaced. |  |  |
|  | Wire Rope Category I If a deficiency constitutes a safety hazard, do not use until replaced. If the deficiency is localized and the problem is corrected by severing the wire rope in two, the undamaged portion may be used. Joining lengths of wire rope by splicing is prohibited. If a rope is shortened, the drum must still have two wraps of wire when the load and/or boom are in the lowest position. |  |  |
|  | Significant distortion of the wire rope structure such as kinking, crushing, unstranding, birdcaging, signs of core failure, or steel core protrusion between the outer strands |  |  |
|  | Significant corrosion |  |  |
|  | Electric arc damage (from a source other than power lines) or heat damage |  |  |
|  | Improperly applied end connections |  |  |
|  | Significantly corroded, cracked, bent, or worn end connections (such as from severe service) |  |  |
|  | Wire Rope Category II If a deficiency is identified, operations involving use of the wire are prohibited until compliance is achieved with the wire rope manufacturer’s established criterion for removal from service, or with a different criterion approved by the manufacturer in writing for that specific wire rope, or the wire rope is replaced. If the deficiency is localized and the problem is corrected by severing the wire rope in two, the undamaged portion may be used. Joining lengths of wire rope by splicing is prohibited. If a rope is shortened, the drum must still have two wraps of wire when the load and/or boom are in the lowest position. |  |  |
|  | Visible broken wires, as follows: |  |  |
|  | In running wire ropes: Six randomly distributed broken wires in one rope lay or three broken wires in one strand in one rope lay where a rope lay is the length along the rope in which one strand makes a complete revolution around the rope |  |  |
|  | In rotation-resistant ropes: Two randomly distributed broken wires in 6 rope diameters or four randomly distributed broken wires in 30 rope diameters |  |  |
|  | In pendants or standing wire ropes: More than two broken wires in one rope lay located in rope beyond end connections and/or more than one broken wire in a rope lay located at an end connection |  |  |
|  | A diameter reduction of more than 5% from nominal diameter |  |  |
|  | Wire Rope Category III If a deficiency is identified, operations involving use of the wire rope in question are prohibited until the wire rope is replaced, or if the deficiency (other than power line contact) is localized and the problem is corrected by severing the wire rope in two, the undamaged portion may continue to be used. Joining lengths of wire rope by splicing is prohibited. Repair of wire rope that contacted an energized power line is also prohibited. If a rope is shortened, the drum must still have two wraps of wire when the load and/or boom are in the lowest position. |  |  |
|  | In rotation-resistant wire rope, core protrusion, or other distortion indicating core failure |  |  |
|  | Prior electrical contact with a power line |  |  |
|  | A broken strand |  |  |
|  | Wire Rope Critical Review Items Give particular attention to the following: |  |  |
|  | Rotation-resistant wire rope in use |  |  |
|  | Wire rope being used for boom hoists and luffing hoists, particularly at reverse bends |  |  |
|  | Wire rope at flange points, crossover points, and repetitive pickup points on drums |  |  |
|  | Wire rope at or near terminal ends |  |  |
|  | Wire rope in contact with saddles, equalizer sheaves, or other sheaves where rope travel is limited |  |  |
|  | Deficiencies found during the annual inspection that require monitoring: |  |  |